OEM Service Excerpts for Line 4 Equipment

KCP-65-160 pump, RF36 filler, DV-4103 pneumatic valve, FRL-4103 regulator, and PHE-4104 limits

# Kestrel KCP-65-160 centrifugal pump service limits

| Item | Normal value | Warning value | Stop and inspect value | Technician note |
| --- | --- | --- | --- | --- |
| Bearing housing temperature | below 70 C after warm-up | 75 C sustained for 10 minutes | 85 C sustained or rapid rise | Measure at bearing housing, not guard surface. Confirm tag location before assigning the reading. |
| Overall vibration, small centrifugal pump class | below 4.5 mm/s RMS | 4.5 to 7.1 mm/s RMS | above 7.1 mm/s RMS if repeatable at steady speed | Dominant 1x plus elevated axial commonly indicates alignment, soft foot, pipe strain, or coupling issue. |
| Mechanical seal leakage | dry or occasional film during start-up only | visible mist or repeat wetting after wipe | continuous drip or product leak to floor | Do not claim replacement unless correct seal kit is installed and seal chamber is leak-tested. |
| Suction strainer differential | below 0.10 bar clean | 0.10 to 0.25 bar | above 0.25 bar or suction below -0.35 bar gauge | Cavitation-like noise with high suction vacuum should be checked before replacing pump components. |
| Alignment, 2 pole motor with jaw coupling | parallel offset 0.05 mm cold max | not applicable | angular 0.05 mm per 100 mm cold max | These OEM values override general site alignment tolerances for this pump train. |

# Mechanical seal and bearing service notes

| Task | Requirement | Evidence expected |
| --- | --- | --- |
| Seal kit selection | Use SK-65-160-CARB-SIC for current wet-end with EPDM elastomers. Do not install SK-65-160-TC-SIC unless the wet-end serial confirms the older seal chamber. | Stores issue record, seal box label, fitted seal faces retained or photographed. |
| Seal replacement verification | After fitting, run at normal operating speed for 10 minutes, check seal drain, and record wet or dry condition. | Technician initials, time of leak test, speed, and visual result. |
| Bearing inspection trigger | Inspect drive-end bearing if housing reaches 85 C, if vibration exceeds 7.1 mm/s RMS after alignment correction, or if acceleration spectrum shows bearing defect frequencies. | Temperature route point, vibration spectrum, and grease or contamination observation. |
| Coupling inspection | Replace elastomer spider if cracked, oil-soaked, missing lugs, or producing red dust; replacement alone does not remove alignment requirement. | removed part condition and follow-up alignment reading. |

# RF36 filler and DV-4103 pneumatic limits

| Component | Limit or setpoint | Fault relation | Technician note |
| --- | --- | --- | --- |
| RF-4101 filler valve bank | minimum pilot pressure 4.8 bar during actuation; normal panel supply 5.5 to 5.8 bar | F17 is generated when pilot pressure drops below limit during the valve demand window | A normal plant header does not rule out local FRL or tubing restriction. |
| DV-4103 diverter valve | full stroke less than 360 ms at 5.0 to 6.2 bar pilot pressure | slow return can create reject gate not clear and false reject count | If bypassing the regulator restores stroke time, inspect filter element, bowl water, regulator seat, and downstream tubing before rebuilding the actuator. |
| FRL-4103 local filter regulator | outlet setpoint 5.6 bar with less than 0.2 bar droop during filler valve cycling | filter swelling or bowl water can create dynamic droop with static pressure apparently normal | Drain bowl each shift during high water carryover and replace element if dark, swollen, or restricted. |

# PHE-4104 process pressure-drop limits

| Item | Normal range | Review range | Corrective action range | Technician note |
| --- | --- | --- | --- | --- |
| Product-side differential pressure | 0.35 to 0.55 bar at 18 m3/h | 0.56 to 0.70 bar | above 0.70 bar | High PHE differential can reduce transfer rate but does not explain pump seal leakage, dominant 1x vibration, or local filler pilot-air droop. |
| After mini-CIP verification | return below 0.55 bar when measured near 18 m3/h | repeat if still above 0.60 bar | open for plate inspection if cleaning does not reduce differential | Compare flow rate when interpreting differential pressure. |

# Source priority note

When records conflict, use calibrated measurements and physical transaction records before status-only close-outs. The OEM service limits should be used for technical thresholds unless a more recent manufacturer bulletin is available. Operator notes are useful for sequence and symptoms but must not be used to invent missing numerical readings.