Line 4 Pump and Motor Alignment Report

P-4102 / M-4102 / CPL-4102 laser alignment and baseplate observations

# Work scope and setup

Alignment check was performed on 2026-02-11 between 21:05 and 23:10 under WO-57302 after blue witness marks suggested the motor had shifted on the baseplate. The contractor used a ShaftLine SL-320 laser kit. Machine train: M-4102 motor driving P-4102 centrifugal pump through CPL-4102 jaw coupling. Coupling spacer length measured at 82 mm. Pump was treated as fixed machine for the final correction.

| Reference | Speed class | Parallel offset limit | Angular limit | Status in report |
| --- | --- | --- | --- | --- |
| General site tolerance sheet MT-01 | 1500 to 3000 rpm | 0.15 mm | 0.15 mm per 100 mm | used by contractor for pass/fail |
| OEM excerpt KCP-65-160, section 5.4 | 2 pole motor, flexible jaw coupling | 0.05 mm cold | 0.05 mm per 100 mm cold | available in binder but not used for pass/fail |

# Laser readings

| Step | Vertical offset | Vertical angular | Horizontal offset | Horizontal angular | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial as-found | -0.18 mm | -0.14 mm/100 mm | +0.31 mm | -0.22 mm/100 mm | Clearly outside both tolerance sets. |
| After first shim move | -0.06 mm | -0.04 mm/100 mm | +0.18 mm | -0.13 mm/100 mm | Vertical close to OEM; horizontal still high. |
| Final recorded reading | -0.03 mm | -0.04 mm/100 mm | +0.12 mm | -0.09 mm/100 mm | Passed against general site sheet but failed OEM pump tolerance. |
| Thermal growth allowance applied | 0.00 mm | 0.00 mm/100 mm | none applied | none applied | No documented reason for ignoring horizontal thermal or base movement. |

# Baseplate and soft foot observations

| Location | Finding | Action | Residual issue |
| --- | --- | --- | --- |
| Motor front left foot | 0.08 mm feeler entered after bolt loosened | Added 0.05 mm shim and retightened | Borderline soft foot remains because final lift not rechecked after horizontal move. |
| Motor rear right foot | 0.04 mm lift on loosen test | No shim added | Acceptable if OEM alignment is reached; not the main cause alone. |
| Pump plinth grout edge | hairline crack at south front corner; old paint line displaced by about 1 mm | Photographed and marked with paint pen | Could allow repeat movement under pipe load; needs follow-up. |
| Suction pipe support | support clamp was loose by hand before tightening | Tightened clamp after alignment readings | Tightening after readings may have changed pump strain. No repeat measurement recorded. |

# Contractor close-out note

Contractor close-out marked the job PASS using the general site tolerance sheet. The final reading was not within the OEM pump tolerance for the documented speed class. The report does not prove a satisfactory final alignment for P-4102, because OEM-specific tolerances were stricter and were available in the maintenance binder.

# Technician interpretation limits

The report supports misalignment as a credible contributor to dominant 1x vibration and seal loading.

The report does not by itself confirm bearing damage. Vibration spectrum and temperature history should be used for bearing assessment.

The report should not be treated as a pass unless the evaluator intentionally chooses the general site tolerance over the OEM tolerance and explains why.

Pipe strain could have changed after the suction support clamp was tightened, because no repeat laser reading was documented afterward.