Line 4 Operator Shift Notes

Packaging line running notes, 2026-02-02 to 2026-02-14

# Shift summary

These notes are copied from the Line 4 production handover book and include operator observations, reset actions, sanitation comments, and hand-written time marks. Times are local plant time. Operator notes are useful for sequence and symptoms, but pressure and temperature readings should be checked against instrumented records where available.

| Date / shift | Operator | Line speed | Product / SKU | Note |
| --- | --- | --- | --- | --- |
| 2026-02-02 early | N. Patel | 175 to 190 bpm | 330 ml still beverage, citrus | Diverter sticky on first start after sanitation. Plant air wall gauge looked normal. Manual purge needed before bottles cleared the reject gate. |
| 2026-02-03 late | G. Moore | 198 bpm before stop | 330 ml still beverage, citrus | Pump area sounded rough, like gravel near the coupling guard. Small wet patch below pump by 19:00. Maintenance changed coupling spider and line restarted. |
| 2026-02-04 early | S. Jenkins | 190 bpm | 330 ml still beverage, citrus | Filler stopped on low-air alarm on first start. Header gauge by wall was around 6.5 bar. We restarted at lower speed and finished the order. |
| 2026-02-05 day | N. Patel | 176 to 198 bpm | 330 ml still beverage, citrus | Flow recovered after strainer basket was cleaned. No suction pressure gauge in place then. Pump noise improved but did not disappear. |
| 2026-02-07 night | A. Bright | 205 bpm before stop | 330 ml still beverage, citrus | Reject gate did not clear, then filler gave low-air fault. Plant air was not down elsewhere. Restart required two purge cycles. |
| 2026-02-08 day | G. Moore | 190 to 198 bpm | 330 ml still beverage, citrus | Pump DE side felt hot near guard. Screen trend said filler bearing hot, but the hot metal felt like the transfer pump housing. |
| 2026-02-10 day | S. Jenkins | 178 bpm | 330 ml still beverage, citrus | Maintenance said pump seal job was closed, but pump still had a mist at the guard drain during the afternoon ramp-up. |
| 2026-02-11 early | A. Bright | 182 bpm | 330 ml still beverage, citrus | Wet floor under pump before start. We wiped it and ran at 46 Hz pump command until filler had enough supply. |
| 2026-02-12 night | N. Patel | 200 to 210 bpm | 330 ml still beverage, citrus | Low-air fault returned when filler valves cycled. Local panel gauge bounced. No other line reported compressor trouble. |
| 2026-02-13 day | G. Moore | 202 to 210 bpm | 330 ml still beverage, citrus | After filter element and strainer work, filler air fault stopped and product flow improved. Pump still vibrates more than normal and seal area is damp. |
| 2026-02-14 early | S. Jenkins | 210 bpm | 330 ml still beverage, citrus | No F17 for first 90 minutes after FRL repair. Pump vibration still visible at guard if speed is pushed above 48 Hz. |

# Time-stamped incident notes

| Time | Observed symptom | Action by operator | Operator reliability |
| --- | --- | --- | --- |
| 2026-02-02 06:08 | Reject gate slow return, filler F17 after valve cycle | Stopped filler, called maintenance, manual purge twice | high for sequence, low for exact air pressure |
| 2026-02-03 18:58 | Rattle near P-4102 guard and visible coupling dust | Reduced line speed and waited for maintenance | high for visible dust and rough sound |
| 2026-02-03 19:22 | Small drip at pump guard drain after spider change | Accepted restart because downstream tank was low | high for leak presence, not a seal diagnosis |
| 2026-02-05 09:12 | Filler bowl starved, bottles underfilled, pump sounded gravelly | Held speed at 176 bpm until strainer cleaned | high for sequence, pressure was not measured |
| 2026-02-07 00:21 | Reject gate not home and F17 low-air alarm | Purge, reset, purge again, restart at 190 bpm | high; corroborated by AIR-007 and WO-57258 |
| 2026-02-08 11:40 | Pump housing too hot to keep hand on, screen label confusing | Reported as pump heat despite trend label saying filler | medium; use VIB route point for actual temperature |
| 2026-02-10 14:18 | Seal area wet after seal job supposedly closed | Wiped drain channel, logged repeat wetting | high for leak presence |
| 2026-02-12 01:38 | Local filler panel pressure bouncing during valve bank actuation | Held restart until maintenance watched the gauge | medium; corroborated by AIR trend |
| 2026-02-13 10:42 | Flow recovered after basket clean; pump still shakes at guard | Ran at 202 bpm and reported remaining vibration | high for before/after symptom |

# Handover cautions written by operators

Do not treat plant header pressure as proof that the filler pilot air is healthy. The fault happens at the local panel when the valve bank cycles.

The pump seal close-out in the maintenance system does not match what operators saw on the floor after the job was marked complete.

The pump got quieter after the strainer was cleaned, but the vibration did not return to normal.

The heat trend label was confusing for several days. Operators believed the hot component was the product transfer pump, not the rotary filler gearbox.

Line 4 can run after the FRL repair, but pushing pump speed above 48 Hz still makes the guard visibly shake.